Construction Math Workbook

You will practice measuring, calculating areas and volumes, estimating materials, and budgeting. The class will work in three groups, each focusing on a different part of a house: Walls, Flooring, and Roofing.

Group A: Walls (Framing & Drywall)

The house is a rectangle, 30 ft × 40 ft, with 8 ft tall walls. Windows: four at 3 ft × 5 ft each. Doors: two at 3 ft × 7 ft each.

Tasks:

1. Find the perimeter of the house (all walls added).

2. Find the total wall surface area (perimeter × height).

3. Subtract the area of windows and doors to get the drywall area.

4. If drywall sheets are 4 ft × 8 ft, how many sheets are needed?

5. Find cost if each sheet is $12.

Group B: Flooring (Concrete Slab / Tile)

The floor area is 30 ft × 40 ft. The concrete slab thickness is 6 inches (0.5 ft)

Tasks:

1. Find the floor area.

2. Find the volume of concrete (area × thickness).

3. Convert cubic feet to cubic yards (1 yd³ = 27ft³).

4. If concrete costs $135 per yd³, find the total cost.

5. Extension: If tile is added later, how many 1 ft × 1 ft tiles are needed to cover the floor?

Group C: Roofing (Shingles / Plywood)

The house has a pitched roof (gable style). Each side of the roof is 30 ft wide × 20 ft slope length.

Tasks:

1. Find the area of one side of the roof.

2. Multiply by 2 for both sides.

3. Roofing shingles come

in bundles covering 33 ft².

4. How many bundles are needed?

5. If each bundle costs $28, what is the total roofing cost?

Combined House Budget

Work as a class to combine results from each group.

Tasks:

1. Add up the total material cost for walls, flooring, and roofing.

2. Estimate the total cost for building the shell of the house.

3. Reflect: Why is accuracy important when ordering construction materials?

Reflection Questions

What part of the math was most challenging: perimeter, area, volume, or conversions?

How does teamwork in construction depend on accurate math?

How can these skills apply to jobs outside of construction?