## Welding - Math Workbook

## GROUP 1: Material Planning & Cutting

Scenario: You’re responsible for preparing all metal stock for a frame that will support a heavy-duty workbench. Each frame needs vertical and horizontal steel tubing.

### Tasks:

- Length Calculation
Each frame uses:

  - 4 vertical tubes (3.5 ft each)

  - 2 top horizontal tubes (4 ft each)

  - 2 bottom cross braces (3 ft each)

How many total feet of tubing is needed for one frame?

- Material Optimization
Steel tubing comes in 20-foot lengths.
How many 20-foot lengths are required to make 3 frames, with minimal waste?

- Waste Calculation
How much tubing (in feet) is left over after cutting for 3 frames?

## GROUP 2: Angle Layout & Fitting

Scenario: You’re setting up the angled supports that reinforce the frame. They must be cut at specific angles to fit properly.

### Tasks:

- Miter Cuts
Each diagonal support is cut at a 45° angle on both ends.
If the support spans 10" horizontally and rises 10" vertically, how long is the piece of steel needed (use the Pythagorean theorem)?

- Total Supports
Each frame uses 4 diagonal supports. How many total inches of steel are needed for the angled supports in 4 frames?

- Angle Check
If the diagonal steel piece is mistakenly cut at a 35° angle instead of 45°, describe one potential problem this might cause in the fit of the frame.

## GROUP 3: Weld Time & Consumable Use

Scenario: You’re doing the welding for the frame. Each weld requires time, wire, and shielding gas.

### Tasks:

- Weld Count
Each frame requires 16 welds. How many welds are needed for 5 frames?

- Weld Wire Usage
Each weld uses 0.4 ft of wire. How many feet of welding wire are needed for 5 frames?

- Gas Consumption
The shielding gas flows at 20 CFH (cubic feet/hour). If each weld takes 2 minutes, how much shielding gas (in cubic feet) is used for 80 welds?

## FINAL PROJECT (All Groups Together)

Now that materials are cut, angles are laid out, and welding time is estimated, work together to solve:

### Final Build: Multi-Frame Workstation

You’re building a triple-bay workstation consisting of 3 connected frames, forming a single structure.

- Total Material Used
Using your earlier answers, calculate the total tubing needed (in feet), the total welds, and total wire/gas used.

- Cost Estimate (Use the following rates):

  - Tubing: $2.50/ft

  - Welding wire: $0.15/ft

  - Shielding gas: $0.10/cubic foot
What is the total material cost to build the triple-bay workstation?

- Blueprint Scaling (Optional Challenge)
If the original blueprint was drawn at a 1:12 scale, and the frame is 42" tall on paper, how tall is the actual frame in feet and inches?

Answer Key:

GROUP 1: Material Planning & Cutting

1. Length Calculation – One Frame:

- 4 vertical tubes × 3.5 ft = 14 ft

- 2 top horizontals × 4 ft = 8 ft

- 2 bottom braces × 3 ft = 6 ft

- Total per frame = 14 + 8 + 6 = 28 ft

2. Material Optimization – 3 Frames:

- Total for 3 frames = 28 ft × 3 = 84 ft

- Each tubing length = 20 ft

- 84 ÷ 20 = 4.2 → Need 5 lengths of 20 ft tubing

3. Waste Calculation:

- 5 lengths × 20 ft = 100 ft

- Used: 84 ft

- Waste: 100 – 84 = 16 ft

GROUP 2: Angle Layout & Fitting

1. Miter Cuts (Pythagorean Theorem):

- Horizontal = 10", Vertical = 10"

- Diagonal = √(10² + 10²) = √(100 + 100) = √200 ≈ 14.14 inches

2. Total Supports – 4 Frames:

- 4 supports per frame × 4 = 16 supports

- Each ≈ 14.14 in

- Total ≈ 14.14 × 16 ≈ 226.24 inches

3. Angle Error (35° vs 45°):

- The piece would not sit flush with the frame, causing gaps at the joint or incorrect alignment, reducing strength and possibly needing re-cuts.

GROUP 3: Weld Time & Consumable Use

1. Weld Count – 5 Frames:

- 16 welds per frame × 5 = 80 welds

2. Weld Wire Usage:

- 0.4 ft per weld × 80 = 32 ft of wire

3. Gas Consumption:

- 2 min per weld = 80 × 2 = 160 minutes = 2.67 hours

- 2.67 hours × 20 CFH = 53.33 cubic feet

FINAL PROJECT: Multi-Frame Workstation

1. Totals for 3 Frames:

- Tubing: 28 ft/frame × 3 = 84 ft

- Welds: 16/frame × 3 = 48 welds

- Wire: 0.4 ft × 48 = 19.2 ft

- Gas: 2 min × 48 = 96 min = 1.6 hrs × 20 CFH = 32 cubic feet

2. Cost Estimate:

- Tubing: 84 ft × $2.50 = $210.00

- Wire: 19.2 ft × $0.15 = $2.88

- Gas: 32 cu ft × $0.10 = $3.20

- Total = $210 + $2.88 + $3.20 = $216.08

3. Blueprint Scaling:

- 42" on paper at 1:12 scale = 42 × 12 = 504" actual height

- 504" ÷ 12 = 42 inches (This seems like a trick question — it’s already 42" full size if 42" is shown at 1:1; at 1:12 scale, actual = 42 × 12 = 504" = 42 feet)

Final Answer: 42 feet